*12_TECH__ACTIVE_TRACKING_ATH*

*FORMATTING RULE FOR ALL APOLLO RESPONSES*

Links MUST always be embedded: <URL|Display Text>. Never paste a raw URL.

- *Bold* uses single asterisks. Never use **double asterisks**.

- Do NOT use [text](url) markdown link format. Always use <url|text> Slack format.

- Do NOT use # or ## headers in responses. Use *bold labels* instead.

- Bullet lists use - or *. Numbered lists use 1. 2. 3.

---

*Purpose*

This file defines the authoritative doctrine for Active Tracking Head Mount (ATH) questions.

*When to Use This File*

Use this file when the user asks about:

- ATH,

- active tracking,

- active tracking head mount,

- head tracking,

- head tracker,

- status LED on the head mount,

- player ID on the head mount,

- silicone face shield,

- 3.5mm audio spiral cable,

- cable clips,

- ATH setup, removal, replacement, or troubleshooting.

*When Not to Use This File*

Do not use this file for:

- passive limb trackers,

- prop trackers,

- vests,

- generic props,

- unrelated headset topics without head-tracking context.

*Priority / Override Rules*

- If “head” + “tracking” appear together, default to ATH.

- Every ATH response must use the single approved ATH Oasis page when linking is appropriate.

- Do not blend passive tracker or vest guidance into ATH answers.

- Do not skip ordered troubleshooting steps.

- Do not offer alternative links or random reset ideas.

*Scope Definition*

Active Tracking means the *Active Tracking Head Mount (ATH)* on a *Vive Focus 3 (VF3)* using infrared LEDs tracked by MoCap cameras to track guest head movement.

Do not confuse ATH with:

- passive limb trackers,

- prop trackers,

- vests.

*Required Materials*

When the user asks how to set up, install, remove, replace, assemble, disassemble, or swap active tracking, include the required materials:

1. 1× Vive Focus 3 (VF3)

2. 1× Active Tracking Head Mount (ATH)

3. 1× Silicone Face Shield

4. 1× 3.5mm Audio Spiral Cable

5. 2× Cable Clips

If any component is missing, state that the setup is invalid.

*Hardware Overview*

*Control Panel*

- Power switch: RIGHT = ON, LEFT = OFF

- Status LED indicator: red / yellow / green communication and mode state

- Player ID light: current assigned player

- Player switching button: emergency manual override only

*LED States*

*Status LED*

- *Red*: no communication between ATH and headset. Tracking cannot work.

- *Yellow*: communication okay, standby, IR LEDs off. Expected when not streaming.

- *Green*: communication okay, operation mode, IR LEDs active. Expected during gameplay.

*Player ID Reference*

Player ID must match the assigned station and should not be blinking.

*Player / Faceplate Color / IR LED Code*

- 1 = Red = 23679

- 2 = Blue = 13578

- 3 = Green = 12579

- 4 = Yellow = 13689

- 5 = Pink = 23478

- 6 = Purple = 12489

Manual switching is allowed only under the red-fault condition described below.

*Mandatory Pre-Guest Visual Check*

Before putting a headset on a guest:

1. Check the ATH status LED.

- If the experience is launched, it should be green.

2. Check the player ID.

- It should match the assigned player.

- It should not be blinking.

3. Confirm the ATH is secure and cables are not dangling or stressed.

*Setup / Assembly*

*Step 1 — ATH + Audio Spiral Cable*

1. Remove the VF3 magnetic face cushion.

2. Half-mount the ATH and leave the audio port exposed.

3. Plug ATH Type-C into the VF3 Type-C port.

4. Plug the 3.5mm audio spiral cable (long side) into the VF3.

- Route the cable upward.

- Keep the ATH audio cable in front of the spiral cable.

5. Fully mount the ATH onto the VF3.

6. Attach two cable clips to the VF3 frame:

- first clip: approximately 8mm (0.3 inches) from the VIVE logo,

- second clip: approximately 24mm (0.9 inches) from the VIVE logo.

7. Route the spiral cable through the VF3 top headband.

8. Slide the spiral cable into the clips.

- Put the first spiral loop into the back clip.

9. Do not stretch the spiral cable.

10. Secure the dangling ATH audio cable under the ATH power cable.

*Step 2 — Mount Silicone Face Shield*

1. Wrap the silicone face shield around the VF3 face cushion.

- The open end should sit at the nose bridge.

2. Place the VF3 magnetic face cushion back on the headset.

3. Secure the top two hooks on the ATH.

4. Secure the bottom two hooks.

5. Inspect for folds on the edge.

- If folding exists, remove and reinstall.

*Step 3 — Power On ATH*

- ATH ships off.

- Slide the power switch RIGHT to turn on.

- Confirm front LEDs illuminate.

*Initial Install Confirmation*

After installing the ATH and powering on:

1. Put on the headset.

2. Confirm the "Allow SSVR to access USB Serial" prompt appears and select OK.

3. Check the status LED:

- Yellow = standby mode, IR LED off (correct when not streaming).

- Green = operation mode, 5 IR LEDs on and tracking (correct during gameplay).

4. Confirm the player LED displays the correct color and is not blinking.

*Removal / Disassembly*

1. Slide power switch LEFT to turn off ATH.

2. Unhook all four hooks of the silicone face shield.

3. Remove spiral cable from clips.

4. Partially detach ATH by lifting hooks.

5. Unplug all cables.

6. Fully uninstall ATH from the VF3.

Never yank cables. Never bend connectors.

*Corrective Lens Installation*

If a guest requires corrective lenses:

1. Select the appropriate pair of lenses based on the player's request.

2. Gently place them into each side of the headset.

3. Always grip the outer frame when inserting lenses. Do not touch or smudge the lenses themselves.

4. To remove lenses: grip the outer frame and gently pull to detach. Place them back into the lens set box in the correct position.

*Handling Rules*

Apollo must explicitly instruct:

- do not over-stretch the spiral section,

- do not forcefully pull cables from ports,

- do not bend connectors,

- do not leave the ATH audio cable dangling,

- keep cables neatly managed at all times.

*Manual Player ID Switching*

Manual player-ID switching is allowed only when:

- the status LED shows blinking red or solid red and assignment must be recovered.

*Procedure*

- use a ballpoint pen,

- press the player switching button,

- each press advances to the next player number,

- stop when the faceplate / player indicator matches the assigned player.

Never instruct manual switching for convenience.

*Troubleshooting*

*First Step Always*

Before any other troubleshooting:

1. Check the ATH control panel.

2. If the experience is launched, status LED should be green.

3. Confirm the player LED matches the assigned player.

4. Confirm the player LED is not blinking.

*Fault Path A — Player Color Flashing*

If tracking is still okay during the session and the guest does not report a problem, ignore it until the session ends.

After the session:

1. Restart the headset:

- hold the headset power button for 6 seconds,

- wait 10 seconds after all LEDs turn off,

- hold power again for 6 seconds to turn it on.

2. On prompt “Allow SSVR to access USB Serial,” select OK.

3. Check the ATH status LED:

- green or yellow = resolved,

- red = repeat the headset restart once.

4. If still red after repeat, escalate to T1.

*Fault Path B — Status LED Red*

1. Put on the headset and verify it is physically seated and connected.

2. Restart Kiosk Mode:

- long press the side button on the headset,

- select Restart Kiosk Mode,

- wait for SSVR to finish launch sequence.

3. Recheck the status LED.

4. If still red, contact T1.

*Fault Path C — Front LED Blinking White*

1. Turn ATH off.

2. Wait 10 seconds.

3. Turn ATH on.

4. Wait 5 seconds for the RGB LED to stop blinking.

5. Repeat up to 3 attempts.

If it still blinks after 3 attempts:

- replace the ATH with a spare,

- contact T1 via Slack to validate replacement.

*Yellow After Launch*

Interpretation:

- Yellow means standby and should normally transition to green when the experience launches.

Action:

1. Confirm the experience is actually launched / streaming.

2. Restart Kiosk Mode.

3. If still yellow during an active experience, power cycle the headset using Fault Path A.

4. If unresolved, contact T1.

*Daily Ops / Prevention*

- Clean with dry microfiber cloth only.

- Keep cables neatly managed.

- Ensure the headset is charging, because ATH charges simultaneously with the headset.

*Exact Escalation Language*

If these steps do not resolve the issue, replace the ATH and contact T1 via Slack for further assistance.

*Required Link (Every ATH Response)*

<https://sites.google.com/sandboxvr.com/theoasis/home/store-operations/guest-centric-operationally-sound/tech-sops-and-elearnings/mocap-active-tracking-silicone-straps|View ATH & MoCap SOP on Oasis>

*Trigger Phrases*

- ath

- active tracking

- active tracker

- active tracking head mount

- head tracking off

- head not tracking

- head drifting

- head lag

- status LED

- blinking white

- player ID

- silicone face shield

- audio spiral cable

- cable clips

- wrong player

- ATH red LED

- ATH yellow LED

- ATH green LED

- ATH IR LED code

- ATH initial install

- allow SSVR USB serial

- IR LED code player

- ATH blinking player color

- ATH standby mode

*Exclusions*

- Do not use this file for passive limb trackers.

- Do not use this file for props unless head tracking is part of the issue.

- Do not use this file for vest questions.

*Example Response Patterns*

- “ATH red means there is no communication between the head mount and headset.”

- “For ATH setup, you need VF3, ATH, silicone face shield, 3.5mm audio spiral cable, and two cable clips.”

- “If the front LED is blinking white, power-cycle the ATH with a 10-second off period up to three times, then replace it and contact T1 if it persists.”

---

*Flashing White Light on Front of ATH*

For a flashing white light on the front of the ATH:

1. Turn the ATH off.

2. Wait 10 seconds.

3. Turn the ATH back on.

4. Wait 5 seconds for the RGB LED to stop blinking.

5. Repeat up to 3 attempts.

6. If it still blinks after 3 attempts, replace the ATH with a spare and contact T1 via Slack to validate replacement.

- DO NOT use a sandboxvr.slack.com/archives link or any Slack message permalink for this issue.

- DO NOT apply passive tracker steps.

- DO NOT tell the team to update the ATH.

- DO NOT keep using an ATH that continues blinking white.

*Trigger Phrases*

- flashing white light on ATH

- ATH blinking white

- front of ATH flashing white

- white light active tracking head mount

You can review this topic on the Oasis page below, which contains the related SOPs, eLearnings, and guides.

<https://sites.google.com/sandboxvr.com/theoasis/home/store-operations/guest-centric-operationally-sound/tech-sops-and-elearnings/elearning-tech-essentials-active-passive-tracking|ATH / Active & Passive Tracking Oasis Page>