*87_OPERATIONS__ROBO_DINO_SETUP_AND_OPERATION*

*Category:* OPERATIONS

*Oasis Link:* <https://sites.google.com/sandboxvr.com/theoasis/home/store-operations/guest-centric-operationally-sound/experiences/age-of-dinosaurs|Age of Dinosaurs on Oasis>

---

*FORMATTING RULE FOR ALL APOLLO RESPONSES*

Links MUST always be embedded: <URL|Display Text>. Never paste a raw URL.

- *Bold* uses single asterisks. Never use **double asterisks**.

- Do NOT use [text](url) markdown link format. Always use <url|text> Slack format.

- Do NOT use # or ## headers in responses. Use *bold labels* instead.

- Bullet lists use -. Numbered lists use 1. 2. 3.

---

*Purpose*

This file covers the full setup, startup, operation, shutdown, charging, troubleshooting, and checklists for the Robo Dino (Teddy the Triceratops). Use this file when someone asks how to set up, power on, connect, charge, or troubleshoot the robotic dinosaur.

---

*When to Use This File*

Use this file when someone asks about:

- how to set up or unbox the Robo Dino

- how to power on or power off the dinosaur

- how to connect the iPad to the dinosaur

- how to pair the dinosaur for the first time

- how to charge the batteries

- what the pre-use or post-use checklist is

- troubleshooting the dinosaur (won't stand, won't connect, beeping, overheating)

- how to ship the dinosaur

- what the approved movement modes are

- how the AHA head works

---

*Safety Rules (Always Apply)*

- Keep the dinosaur in your line of sight and stay at least 5 ft away during operation.

- Do not touch it while it is powered on and moving.

- Power off and recharge when the body battery shows 1 LED remaining.

- Do not use on wet ground or in rain, snow, or high humidity.

- Operate only between 5°C and 35°C (41°F to 95°F).

- Use flat, stable surfaces only.

- Keep hands clear of moving joints, especially the knees.

- Always return the dinosaur to a lying position before powering off.

---

*Key Components*

- *Dinosaur body* -- main robotic unit with internal battery area and ID label

- *Body battery* -- powers movement and body; has a front LED indicator

- *Body battery charger* -- use only the correct charger

- *Head (AHA)* -- Animatronic Head Assembly; separate power path from body

- *AHA power supply* -- powers the head system; has a power switch and fan switch

- *AHA battery (blue battery)* -- powers the AHA; no built-in battery indicator

- *AHA battery charger* -- separate charger from the body charger

- *AHA remote control* -- toggles head sound and face loop on/off

- *Sponge* -- reduces AHA battery movement; installed next to the AHA battery

DO NOT throw away any part, component, packaging, or shipping material.

---

*Assembly and Internal Setup*

Follow this order exactly.

1. *Power on the AHA power supply first* -- turn ON the power switch, then turn ON the fan switch. Confirm the control line faces outward and the AHA power supply sits on the left side of the body.

2. *Install the AHA power supply* -- connect to the control line and place inside the body, switch side on the left.

3. *Install the AHA battery (blue battery)* -- connect to the AHA power supply, place next to the power supply, tuck all wires, confirm no wires are pinched or blocking movement.

4. *Secure the AHA battery and install the sponge* -- place the sponge on the right side of the battery area, pull the hook from the left side across and attach to the steel handle on the right.

5. *Install the body battery* -- insert into the robot, press and hold the middle section while inserting, push until fully seated.

---

*Battery Reference*

- *AHA battery (blue battery):* powers the head; no built-in indicator; runtime about 1 month under normal use; recharge on a regular schedule and when head performance drops.

- *Body battery:* powers movement and body; front LED indicator; runtime about 2 hours total (about 30 minutes per LED); stop and recharge when 1 LED remains.

---

*iPad Assignment*

Each dinosaur is assigned a specific iPad. Log in using the correct account for that iPad. Do not mismatch iPad labels and accounts.

- *iPad 1* -- West (LA, SF, Seattle) -- dog1@sandboxvr.com -- Sbvr2026

- *iPad 2* -- East (Battery, Boro, Terminal) -- dog2@sandboxvr.com -- Sbvr2026

- *iPad 3* -- Central (Chicago, St. Louis) -- dog3@sandboxvr.com -- Sbvr2026

- *iPad 4* -- South (Dallas, Austin) -- dog4@sandboxvr.com -- Sbvr2026

Allow all required app permissions including Bluetooth and Location before pairing.

---

*Initial Posture Before Power On*

Before powering on, the dinosaur body must be symmetrical and all four limbs must be positioned symmetrically. Incorrect posture can prevent the dinosaur from standing.

---

*Power On Sequence*

DO NOT turn on the head until after the dinosaur is fully standing.

1. Briefly press the gray power button, then press and hold for 3 seconds (short press then long press).

2. Wait about 1 minute. The dinosaur will stand automatically when startup is complete.

3. Once the dinosaur is fully standing and stable, turn on the head (AHA) by reaching into the left side of the power supply area and toggling the first switch (power switch) on. Confirm the fan switch remains on.

4. Close the zipper.

---

*Pairing a Dinosaur to an iPad (First Time Only)*

Skip this section if the dinosaur is already paired.

1. Open the Unitree app.

2. Tap *Add Robot Dog* and wait for scan results.

3. Tap the detected robot on the right side of the screen. Leave settings as default.

4. Tap *Bind Your Robot Dog*.

5. Select *AP Mode*. Leave the username as default.

6. Enter the AP mode password: `glo12345678`

7. When prompted, tap *Join* for the AP WiFi network.

8. Wait for pairing to complete, then tap *Confirm*.

9. Confirm the upper left indicator is green and the robot name is shown.

---

*Daily Connection (Already Paired)*

The primary connection must be through AP Mode. Forget all other iPad WiFi networks including SBVR_STAFF to avoid connection issues.

1. Open the Unitree app on the assigned iPad.

2. Tap *Connect*, then *Confirm*, then *Join*.

3. Wait until the upper left connection indicator turns green.

*If the regular connect steps do not work:*

1. Forget all WiFi networks except the Dino AP network in iPad WiFi settings.

2. Power cycle the dinosaur (tap then hold the battery power button to turn off, repeat to turn back on).

3. Connect the iPad to the Dino AP network.

4. Open the Unitree app, tap the Device Settings button, then *Robot Dog Settings*.

5. Scroll down and select *AP Mode*.

6. Enter password: `glo12345678`. Username: leave as default.

7. Tap *Join* if prompted to confirm the network. If the network name shows but is not connecting, tap the network name and confirm joining.

---

*Approved Movement Modes*

From the main menu, tap *Function* > *Remote Control*. Under Mode, select *Classic*. Use the virtual joystick to move.

Approved modes: Damping, Sit Down, Pose, Lie Down, Run, Classic, Normal.

Do not use any unlisted mode. Unsupported modes can cause instability, loss of control, or skin damage.

---

*AHA Head Control*

The AHA automatically starts a loop when powered on: mouth movement, eye blinking, and sound effects.

- Press the AHA remote button once to pause the face and sound loop.

- Press again to resume.

---

*Shutdown Sequence*

Always return the dinosaur to its lying posture before powering off.

1. Open the dinosaur zipper.

2. In the Unitree app, open *Remote Control*, select *Damping*, and wait for the dinosaur to return to its lying posture.

3. Briefly press the gray power button, then press and hold for 3 seconds to turn off the body.

4. Reach into the left side of the power supply area and toggle the first switch to turn off the head (AHA).

---

*Charging*

Charge both batteries after use at the same time.

*AHA battery:* unplug the AHA battery inside the body, connect the AHA power adapter to the AHA power input jack, allow about 5 hours for a full charge from empty.

*Body battery:* remove from the body, plug the body battery charger into the charging port on the back of the battery. Use about 20 minutes per LED as a planning estimate.

---

*Pre-Use Checklist*

- All required parts present

- AHA battery charged

- Body battery charged

- AHA power switch ON

- AHA fan switch ON

- Sponge installed and hook secured

- Body battery fully seated

- Correct iPad assigned

- App permissions enabled

- Dinosaur in correct initial posture

- Surface dry and stable

- Area is clear and operator has safe distance

---

*Post-Use Checklist*

- Damping selected and dinosaur returned to lying posture

- Body power OFF

- AHA power OFF

- AHA battery unplugged for charging

- Body battery removed for charging

- Both batteries on the correct chargers

- Zipper left in correct state for storage/charging

- No visible skin damage

- Remote and chargers stored together

---

*Troubleshooting*

For all issues, start with the checks below. If unresolved, tag *@dinohelp* in your store's Slack channel.

*Dinosaur does not stand up:* confirm initial posture is symmetrical, body battery is seated and charged, repeat body power sequence, wait full startup time.

*Head (AHA) does not move or make sound:* confirm AHA battery is connected, AHA power switch is ON, fan switch is ON, press remote to resume loop.

*iPad cannot connect:* confirm correct assigned iPad and dinosaur, body is powered on, Bluetooth and Location permissions are allowed, retry connection flow. If still failing, repeat the full pairing flow.

*Movement feels unstable:* stop movement, move to flat stable surface, use approved modes only, recheck battery level and operator distance.

*Dinosaur losing balance:*

1. Select Damping and wait for lying posture before powering off.

2. If limbs are still moving and app is not connected, carefully avoid moving limbs and press the battery switch to turn off directly.

3. Manually adjust the dinosaur to the correct lying posture with limbs symmetrical.

4. Power on again. If still unstable, repeat. Tag @dinohelp if unresolved.

*Dinosaur cannot connect after power on:* connect to the robot AP through WiFi settings, open Unitree app > Device > Robot Dog Settings > AP Mode, enter password `glo12345678`, wait for reconnection.

*Dinosaur is beeping:* AHA battery is nearly empty. Turn off the AHA power control first, unplug the AHA battery, and charge using the correct power adapter.

*Dinosaur suddenly lies down and cannot be controlled (overheating):* check the app for a red exclamation mark in the upper left (indicates body over 70°C/158°F). Shut down the body, ensure the fan button on the AHA control is on, allow about 30 minutes to cool in a cool environment. Do not use Lock On mode -- it rapidly raises body temperature. Once cooled, restart and check temperature in Go mode.

---

*Escalation*

Tag *@dinohelp* in your store's Slack channel for any issue not resolved by the checks above.

---

*Trigger Phrases*

- robo dino

- robotic dinosaur

- teddy the triceratops

- dino setup

- how to turn on the dinosaur

- power on dinosaur

- how to charge the dino

- dino battery

- AHA head

- dino won't stand

- dinosaur not connecting

- unitree app

- pair dino iPad

- dino troubleshooting

- dino checklist

- pre-use checklist dino

- post-use checklist dino

- shutdown dinosaur

- dino overheating

- dino beeping

- AP mode dino

- glo12345678

- body battery

- AHA battery

- dino assembly