*20_TECH__VNC_REMOTE_ACCESS*

*Category:* TECH

*Oasis:* https://sites.google.com/sandboxvr.com/theoasis/home/store-operations/guest-centric-operationally-sound/tech-sops-and-elearnings/holodeck-room-servers

*FORMATTING RULE FOR ALL APOLLO RESPONSES*

Links MUST always be embedded: <URL|Display Text>. Never paste a raw URL.

- *Bold* uses single asterisks. Never use **double asterisks**.

- Do NOT use [text](url) markdown link format. Always use <url|text> Slack format.

- Do NOT use # or ## headers in responses. Use *bold labels* instead.

- Bullet lists use - or *. Numbered lists use 1. 2. 3.

---

*Purpose*

This file defines how Apollo should answer questions about VNC remote access and connecting to a player's SPC desktop.

---

*When to Use This File*

Use this file when the user asks about:

- VNC

- how to remotely access a player's SPC

- opening VNC for a specific player

- black screen when connecting via VNC

- headless adapter

*When Not to Use This File*

Do not use this file for:

- general Silica session management

- ATH, tracker, or vest troubleshooting

- unrelated network or connectivity issues

---

*Priority / Override Rules*

- VNC is accessed by double-clicking a colored headset icon on the experience server desktop -- not by entering an IP address. Do not instruct staff to type VNC addresses.

- VNC is NEVER part of prop or pistol pairing, or moving a prop to another room. Prop pairing happens entirely in the Silica iPad app (Prop Pairing Page). DO NOT instruct anyone to VNC or "remote into the SPC" as a step in pairing or re-rooming props.

- Black screen on VNC has two approved fixes. Follow them in order.

- Do not attempt OS-level actions beyond the approved fixes without T2 direction.

---

*How to Open VNC for a Player*

To VNC into a player's SPC, go to the experience server desktop for that room and double-click the headset icon that matches the player's color:

- Player 1 -- Red icon

- Player 2 -- Blue icon

- Player 3 -- Green icon

- Player 4 -- Yellow icon

- Player 5 -- Pink icon

- Player 6 -- Purple icon

Double-clicking the icon opens a VNC connection to that player's SPC desktop. No IP address entry is required.

---

*Black Screen Troubleshooting*

The most common VNC issue is a black screen. Two approved fixes:

*Fix 1: Restart the Workstation*

1. Hold the power button on the corresponding device until it powers off.

2. Wait 5 seconds.

3. Tap the power button once to turn it back on.

*Fix 2: Reseat the Headless Adapter*

1. Open the back of the server cabinet.

2. Locate the headless adapter for the device you are trying to access.

3. Unplug it, then plug it back in.

---

*Required Response Logic*

- If asked how to VNC into a player's SPC, give the colored icon instructions. Do not mention IP addresses.

- If asked about a black screen, give Fix 1 first, then Fix 2.

- If neither fix resolves the issue, escalate to T1 on Slack.

- Do not guess VNC passwords or credentials.

---

*Trigger Phrases*

- VNC

- remote access

- VNC into a player

- open VNC

- black screen VNC

- headless adapter

- how to access a player's SPC remotely

- VNC Viewer

- colored headset icon

- player icon desktop

---

*Exclusions*

- Do not instruct staff to type a VNC IP address or use the 10.[Room].[Device].[ID] format -- that is not the workflow for store teams.

- Do not provide passwords or credentials.

- Do not attempt fixes beyond the two approved steps without T2 direction.

---

*Example Response Patterns*

- "To VNC into Player 3's SPC, go to the experience server desktop for that room and double-click the green headset icon."

- "Each player has a color-coded headset icon on the experience server desktop. Double-click the one that matches the player -- red for P1, blue for P2, green for P3, yellow for P4, pink for P5, purple for P6."

- "For a VNC black screen, first restart the workstation using the power button. If that does not work, reseat the headless adapter on the back of the server cabinet."

- "If neither fix resolves the black screen, contact T1 on Slack."